Navigator development explanation

1. coordinate system

- Structure:

[Latitude, Longitude, Elevation] (Lunar Spherical Coordinates)

- Why [Lat, Lon, Alt]over 3D Cartesian (X, Y, Z)?

Direct compatibility with NASA Lunar DEM (Digital Elevation Model)data and web 3D GIS engines (e.g.,CesiumJS,Three.js) using standard GeoJSON format ([Lon, Lat, Alt]).

Enables instant calculation of surface horizontal distances and vertical elevation differences to evaluate slope angles ($\theta$) in real-time.

2. route calculations

1) Oxygen Used

- Oxygen Consumption Model

Formula:

Base O2 Rate (L/min)= 0.8 * Vehicle Oxygen Factor

Slope Penalty= 1 + (theta / 10)

Total Oxygen Used (Liters)= Pure Travel Time (min) * Base O2 Rate * Slope Penalty

Physiological Basis:

Uses 0.8 L/minbaseline astronaut metabolic consumption (NASA-STD-3001, Vol 2 / NASA HIDP).

2) Time

- Terrain-Adaptive Travel Time

Formula:

Effective Speed (km/h)= max(Base Speed * cos(theta),0.1)

Pure Travel Time (min)= (Distance (km) / Effective Speed) * 60

Total Travel Time (min)= Pure Travel Time + (Stops Visited * 15 min)

Physiological Basis:

Incorporates a cosine speed attenuation model for slope climbing ($\theta$),including a 15-minute operational overhead for station stops.

3) Elevation

- Elevation & Topography

Tracks slope angles ($\theta$) and profile shifts across crater rims and ridges to dynamically alter speed and exertion penalties.

4) Possible Stops along the way (oxygen stations, other bases, etc)

=> oygen station, power station, communication hub

- Waypoints & Strategic Stops (NASA 13 Candidate Regions) Categorized from the NASA Artemis III 13 Candidate Landing Regionsbased on USGS terrain data for resource availability:

a) OXYGEN_STATION (4 Sites):

Water-ice processing in Permanently Shadowed Regions (PSR)
=> Peak Near Shackleton, Haworth, Faustini Rim A, Amundsen Rim

b) POWER_STATION (4 Sites):

High-altitude solar power depots ($80\sim90\%+$continuous illumination)
=> Malapert Massif, Leibnitz Beta Plateau, Mons Mouton, Mons Mouton Plateau

c) COMMUNICATION_HUB (5 Sites):

High-ridge Direct-to-Earth (DTE) communication line-of-sight
=> Connecting Ridge, Connecting Ridge Ext, de Gerlache Rim 1 & 2, Nobile Rim 1

5) Vehicle Being Used

- Vehicles & Mobility Parameters

| Vehicle Type | NASA Reference | Speed | O2 Factor | Key Features & Constraints |
| --- | --- | --- | --- | --- |
| xEMU Suit(Walking) | Axiom / Apollo EVA Data | 3.5 km/h | 1.0 (100%) | Full physical exertion; high O2 consumption on slopes (≤ 15°). |
| Unpressurized Rover | Artemis LTV | 12 km/h | 0.4 (40%) | Open-top driving; reduces astronaut exertion by 60% (≤ 20°). |
| Pressurized Cruiser | JAXA / MPH Habitat | 20km/h | 0.0 (0%) | Enclosed life support cabin; zero suit O2 deduction (≤ 25°). |
| Lunar Hopper | Intuitive Machines / Lander | 60km/h | 0.0 (0%) | Rocket-powered sub-orbital flight; bypasses slope slowdowns. |

3. hazard detection

1) slope

Triggered when incline_deg > max_incline.Drastically reduces speed and applies route risk penalties to prevent vehicle rollover.

2) PSR / Cold

Triggered in Permanently Shadowed Regions (is_psr == True).Warns of cryogenic conditions and extreme battery drain risks.

3) Comm Blackout

Triggered when DTE Line-of-Sight is lost (has_dte_los == False).Prioritizes rerouting through designated COMMUNICATION_HUBwaypoints.

4) Dust / Abrasion

Triggered when cumulative travel exceeds 10km.Recommends stopping at maintenance depots to mitigate mechanical joint and seal wear.

5) Radiation

Triggered when continuous sunlight exposure exceeds 4 hours.Flags high radiation dosage risks during peak EVA operations.

4. References : Official NASA Data References

NASA-STD-3001, Vol 2:

Spaceflight Human System Standard: Human Health and Performance

NASA HIDP:

Human Integration Design Processes (EVA Metabolic Rates & O2 Consumption Studies)

NASA Artemis LTV Requirements Document:

Lunar Terrain Vehicle Mobility Specifications

5. Code Overview

1. Constants & NASA Reference Data

* `BASE_O2_CONSUMPTION_L_PER_MIN = 0.8`: Standard astronaut metabolic oxygen consumption rate (0.8 L/min).

* `VEHICLE_SPECS`: Mobility parameters for 4 lunar vehicles (xEMU Suit, Unpressurized Rover/LTV, Pressurized Cruiser, Lunar Hopper) including speed, O2 reduction factors, and max slope angle limits.

* `WAYPOINTS_DB`: Coordinates and station types (`OXYGEN_STATION`, `POWER_STATION`, `COMMUNICATION_HUB`) derived from NASA’s 13 Artemis III candidate landing regions.

2. Navigation & Life-Support Calculator (`LunarNavigator`)

- `calculate_effective_speed()`: Computes terrain-adaptive speed reduction based on slope angles ($\text{Base Speed} \times \cos\theta$).

- `calculate_travel_time()`: Calculates pure travel time and total travel time, incorporating a 15-minute operational overhead per visited waypoint.

- `calculate_oxygen_used()`: Evaluates total O2 consumption by applying metabolic rate, vehicle factor, and slope penalty.

3. 5-Stage Hazard Detection System (`detect_hazards`)

- Identifies 5 critical surface hazards (**Slope, PSR/Cold Trap, Comm Blackout, Dust/Regolith Abrasion, Radiation**) and returns status warnings with rerouting/maintenance action steps.

4. Execution Demo & Test Suite (`__main__`)

- A sample simulation block demonstrating route calculations and hazard detection output. Run `python luna_navigator.py` directly to see the demo output in your terminal.